IoT and Edge AI
Practical Activity Challenges

Beginner to Advanced

A readable, classroom-ready challenge guide for Arduino, IoT, TinyML and Edge AI

# How to Use This Guide

The challenges progress from basic Arduino input/output to sensors and actuators, then to automation, IoT connectivity, data preparation, TinyML/Edge AI and finally integrated intelligent systems. Beginner tasks can include starter code. As students progress, provide the problem and requirements but let them design the circuit, logic and testing method.

## Learning Path

Level 1 - Beginner: Arduino, digital/analog input, PWM, servo, motor, DHT11 and IR sensing.

Level 2 - Intermediate: combine sensors, actuators, states, timing and multi-sensor logic.

Level 3 - IoT: connect devices to dashboards and implement remote monitoring/control.

Level 4 - Edge AI: collect data, clean it, select features, train/evaluate models and run inference locally.

Level 5 - Advanced: combine Edge AI and IoT for resilient, efficient and intelligent systems.

# Beginner Level

Focus: understanding the microcontroller, simple input/output, sensing and actuator control.

# Challenge 1: Arduino Digital Output

LEVEL: BEGINNER

DESCRIPTION Program an Arduino to control an LED and use timing instructions to create visible output patterns. This is the first step in understanding how a microcontroller controls a physical device.

## Learning Objectives

Identify an Arduino digital output pin.

Use pinMode(), digitalWrite() and delay().

Relate program instructions to physical output behavior.

## Student Challenge

Build an LED circuit, blink the LED, then create a unique pattern containing at least three different ON/OFF durations.

## Expected Evidence / Output

Working circuit

Arduino sketch

Short explanation of how changing delay values changes the pattern

## Extension Challenge

Replace delay-based timing with a non-blocking approach when students are ready.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 2: Push Button Control

LEVEL: BEGINNER

DESCRIPTION Introduce digital input by allowing a physical push button to control an LED. Students move from producing an output to reading the environment through a simple user input.

## Learning Objectives

Configure a digital input.

Use conditional statements.

Understand HIGH/LOW states and pull-up/pull-down behavior.

## Student Challenge

Make the LED respond to a push button, then modify the program so one press toggles the LED state instead of requiring the button to remain pressed.

## Expected Evidence / Output

Working button-controlled LED

Code with an if/else decision

Demonstration of toggle behavior

## Extension Challenge

Investigate switch bouncing and implement simple debouncing.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 3: Analog Sensor Reading

LEVEL: BEGINNER

DESCRIPTION Students read a continuously varying analog value, such as a potentiometer, and inspect the numerical result in the Serial Monitor.

## Learning Objectives

Understand analog-to-digital conversion.

Use analogRead().

Interpret raw ADC values.

## Student Challenge

Connect a potentiometer or available analog sensor and record minimum, middle and maximum readings. Explain what the numbers represent.

## Expected Evidence / Output

Serial Monitor readings

Three recorded test values

Brief interpretation of the ADC range

## Extension Challenge

Convert the raw reading into a percentage or engineering unit.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 4: PWM LED Brightness

LEVEL: BEGINNER

DESCRIPTION Students connect analog input to controlled output by mapping a potentiometer reading to LED brightness using PWM.

## Learning Objectives

Explain PWM at a basic level.

Use analogWrite() on a PWM-capable pin.

Map an input range to an output range.

## Student Challenge

Use a potentiometer to smoothly control LED brightness. Test at least five input positions and record the corresponding output values.

## Expected Evidence / Output

Working brightness controller

Code using input-to-output mapping

Five-value test table

## Extension Challenge

Create automatic fading without a potentiometer.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 5: Servo Position Control

LEVEL: BEGINNER

DESCRIPTION Using the servo concept already introduced in the source slides, students command precise angular positions and observe the relationship between software values and mechanical position.

## Learning Objectives

Connect a servo safely.

Use a servo library.

Command angular position.

## Student Challenge

Move the servo to 0°, 45°, 90°, 135° and 180°, pause at each position, and then return to 0°.

## Expected Evidence / Output

Working servo circuit

Programmed five-position sequence

Observation of any positioning error

## Extension Challenge

Use an analog input to control servo angle continuously.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 6: DC Motor Direction Control

LEVEL: BEGINNER

DESCRIPTION Students use a motor driver rather than powering a motor directly from the microcontroller. They learn why higher-current actuators need an interface stage.

## Learning Objectives

Explain the purpose of a motor driver.

Control motor start/stop and direction.

Apply safe separation of logic and motor power.

## Student Challenge

Make a DC motor rotate clockwise, stop, rotate anticlockwise and stop again using a programmed sequence.

## Expected Evidence / Output

Correct motor-driver wiring

Working direction-control program

Explanation of why the motor is not driven directly from an I/O pin

## Extension Challenge

Add push buttons for manual direction control.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 7: DC Motor Speed Control

LEVEL: BEGINNER

DESCRIPTION Students extend motor control by using PWM to create multiple speed levels.

## Learning Objectives

Use PWM for actuator control.

Relate duty cycle to motor behavior.

Test and compare speed settings.

## Student Challenge

Program OFF, low, medium and high motor-speed states and demonstrate each state for a fixed period.

## Expected Evidence / Output

Four-state motor demonstration

Code with clearly labelled speed values

Observation table describing motor response

## Extension Challenge

Control speed from a potentiometer.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 8: DHT11 Environmental Sensor

LEVEL: BEGINNER

DESCRIPTION Building on the DHT11 activity in the supplied slides, students acquire real temperature and humidity data and display it for observation.

## Learning Objectives

Connect and read a digital environmental sensor.

Use a sensor library.

Interpret temperature and relative humidity.

## Student Challenge

Read temperature and humidity every two seconds and record ten consecutive samples. Identify whether the readings are stable or changing.

## Expected Evidence / Output

Working DHT11 circuit

Ten-sample data table

Serial output showing both variables

## Extension Challenge

Calculate minimum, maximum and average values.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 9: IR Object Detection

LEVEL: BEGINNER

DESCRIPTION Students use the IR sensor introduced in the slides to detect the presence or absence of an object and convert that detection into a visible output.

## Learning Objectives

Understand binary object detection.

Read a sensor state.

Connect sensing to an indicator.

## Student Challenge

Turn an LED ON when an object is detected and OFF when no object is present. Test the sensor at different distances.

## Expected Evidence / Output

Working IR detector

Code and circuit

Short table of detection results at different distances

## Extension Challenge

Adjust sensor sensitivity and determine a reliable operating distance.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 10: Sensor-Based Buzzer Alert

LEVEL: BEGINNER

DESCRIPTION Students create their first autonomous decision system: a sensor value is evaluated locally and an actuator is triggered when a threshold is crossed.

## Learning Objectives

Use threshold logic.

Combine sensor input and actuator output.

Explain local decision-making.

## Student Challenge

Use temperature as the input. Activate a buzzer or warning LED when the measured temperature exceeds a chosen threshold and reset the alert when it falls below the threshold.

## Expected Evidence / Output

Working alert prototype

Threshold-based code

Five test cases showing input and output state

## Extension Challenge

Add separate Warning and Critical thresholds.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Intermediate Level

Focus: integrating components into autonomous systems with more complex decision logic.

# Challenge 11: Automatic Temperature Fan

LEVEL: INTERMEDIATE

DESCRIPTION Students combine DHT11 sensing with DC motor PWM to build a simple automatic cooling controller. The system reacts without continuous human input.

## Learning Objectives

Integrate a sensor and actuator.

Implement multi-level decision rules.

Use PWM in an automated system.

## Student Challenge

Implement: below 25°C fan OFF, 25–30°C low speed, above 30°C high speed. Students may adjust thresholds after testing.

## Expected Evidence / Output

Working automatic fan

Flowchart of decision logic

Test table covering all three states

## Extension Challenge

Add hysteresis to prevent rapid switching near a threshold.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 12: Smart Servo Ventilation

LEVEL: INTERMEDIATE

DESCRIPTION A servo represents a window or ventilation flap. Environmental readings determine how far the vent opens.

## Learning Objectives

Map environmental conditions to mechanical position.

Design multi-state control logic.

Explain actuator selection.

## Student Challenge

Create at least four ventilation states, such as closed, slightly open, half open and fully open, based on temperature.

## Expected Evidence / Output

Servo ventilation prototype

State/angle table

Code implementing the states

## Extension Challenge

Use both temperature and humidity in the decision.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 13: Automatic Door

LEVEL: INTERMEDIATE

DESCRIPTION An IR sensor detects approach and a servo acts as a door mechanism. Students integrate event detection, timing and actuation.

## Learning Objectives

Build an event-driven system.

Coordinate sensor, servo and timing.

Consider safe default states.

## Student Challenge

Open the servo door when an object is detected, keep it open for a defined period and close it when the path is clear.

## Expected Evidence / Output

Working miniature door

Program logic/flowchart

Demonstration under at least five approach events

## Extension Challenge

Prevent closing while an object remains detected.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 14: IR Object Counter

LEVEL: INTERMEDIATE

DESCRIPTION Students transform repeated IR detection events into useful information by counting objects that pass a sensing point.

## Learning Objectives

Use variables to maintain state.

Detect transitions rather than continuous presence.

Display accumulated information.

## Student Challenge

Count each passing object once and show the total in Serial Monitor. Demonstrate at least 20 passes without double-counting.

## Expected Evidence / Output

Object counter program

20-event test record

Explanation of how double counting was avoided

## Extension Challenge

Add a reset button or local display.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 15: Occupancy Indicator

LEVEL: INTERMEDIATE

DESCRIPTION Students convert detection into a meaningful room/space status.

## Learning Objectives

Model a system using states.

Use visual indicators.

Translate sensor data into user information.

## Student Challenge

Display green for Available and red for Occupied based on IR detection. Add a short delay/filter so brief noise does not constantly change the state.

## Expected Evidence / Output

Two-state indicator

State diagram

Noise-filtering explanation

## Extension Challenge

Add a third state such as Unknown/Sensor Fault.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 16: Environmental Warning System

LEVEL: INTERMEDIATE

DESCRIPTION Temperature and/or humidity values are classified into Normal, Warning and Critical categories. This introduces classification using explicit rules before machine learning.

## Learning Objectives

Design class boundaries.

Implement multi-class logic.

Test boundary conditions.

## Student Challenge

Define three environmental classes, control different indicators for each, and test values close to every boundary.

## Expected Evidence / Output

Class definition table

Working three-state prototype

Boundary test results

## Extension Challenge

Use a buzzer pattern that differs for Warning and Critical.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 17: Smart Cooling Controller

LEVEL: INTERMEDIATE

DESCRIPTION Students use two environmental variables rather than one, requiring more thoughtful decision logic.

## Learning Objectives

Combine multiple sensor variables.

Design AND/OR conditions.

Explain why multiple features can improve decisions.

## Student Challenge

Control a fan using both temperature and humidity. Students must justify their rule set rather than copy fixed thresholds.

## Expected Evidence / Output

Decision table

Working controller

Written justification for chosen rules

## Extension Challenge

Create a calculated comfort index before making the decision.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 18: Manual and Automatic Motor Control

LEVEL: INTERMEDIATE

DESCRIPTION Students design a system with two operating modes: human control and autonomous sensor-based control.

## Learning Objectives

Implement mode selection.

Separate manual and automatic logic.

Design predictable state transitions.

## Student Challenge

Use a switch/button to choose Manual or Automatic mode. Manual mode uses a potentiometer for speed; Automatic mode uses sensor conditions.

## Expected Evidence / Output

Two-mode working system

Mode-state diagram

Demonstration of safe switching between modes

## Extension Challenge

Remember the last manual setting when returning from automatic mode.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 19: Sensor-to-Servo Mapping

LEVEL: INTERMEDIATE

DESCRIPTION Students convert a measured quantity into proportional actuator movement instead of fixed states.

## Learning Objectives

Scale numerical ranges.

Calibrate input and output.

Recognize saturation and limits.

## Student Challenge

Map a selected sensor range to servo angles from 0° to 180°. Prevent values outside the expected range from commanding unsafe positions.

## Expected Evidence / Output

Calibrated mapping code

Input/angle test table

Explanation of range limiting

## Extension Challenge

Compare proportional mapping with discrete angle states.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 20: Multi-Sensor Alarm

LEVEL: INTERMEDIATE

DESCRIPTION Students create simple sensor fusion by requiring evidence from more than one input before producing an alarm.

## Learning Objectives

Combine independent sensor signals.

Reduce false alarms using logical conditions.

Create a multi-input decision table.

## Student Challenge

Trigger an alarm only when a selected environmental condition and an occupancy/object condition occur together. Test all combinations of inputs.

## Expected Evidence / Output

Truth/decision table

Working multi-sensor alarm

Test evidence for every input combination

## Extension Challenge

Create separate alarms for single-sensor and combined conditions.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# IoT Level

Focus: networking, dashboards, remote control, logging and event-driven communication.

# Challenge 21: IoT Temperature Monitor

LEVEL: IOT

DESCRIPTION Students move from local Serial Monitor output to networked monitoring by publishing environmental readings to a remote dashboard using suitable network-capable hardware.

## Learning Objectives

Explain the basic IoT data path.

Transmit sensor measurements.

Distinguish local sensing from remote visualization.

## Student Challenge

Send temperature and humidity readings to an IoT dashboard at a sensible interval and verify that the remote values match local readings.

## Expected Evidence / Output

Live dashboard

Device code

Comparison of local and remote readings

## Extension Challenge

Add timestamps and signal/device status.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 22: Remote Environmental Dashboard

LEVEL: IOT

DESCRIPTION Students organize sensor information into a useful monitoring interface rather than simply transmitting raw values.

## Learning Objectives

Design readable visualizations.

Interpret time-series data.

Select meaningful dashboard indicators.

## Student Challenge

Create dashboard elements for current temperature, current humidity and recent trends. Add a visual warning when a limit is exceeded.

## Expected Evidence / Output

Dashboard screenshots

Trend visualization

Explanation of chosen widgets/indicators

## Extension Challenge

Add daily minimum, maximum and average values.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 23: Remote LED Control

LEVEL: IOT

DESCRIPTION Students implement the opposite IoT direction: commands travel from a user interface to the physical device.

## Learning Objectives

Explain cloud-to-device control.

Handle remote commands.

Maintain a known output state.

## Student Challenge

Control an LED remotely and report the actual LED state back to the interface so the dashboard reflects the physical device.

## Expected Evidence / Output

Remote control demonstration

Command and status logic

Evidence of state feedback

## Extension Challenge

Add authentication or a local override button.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 24: Remote Servo Control

LEVEL: IOT

DESCRIPTION A remote interface sends a desired actuator position to the edge device.

## Learning Objectives

Transmit command values.

Validate remote inputs.

Control a physical actuator from a network interface.

## Student Challenge

Provide remote controls for servo angle. Reject or constrain invalid angles and display the commanded/current position.

## Expected Evidence / Output

Remote servo interface

Input validation in code

Successful tests at multiple angles

## Extension Challenge

Add preset positions such as Closed, Half and Open.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 25: Smart Fan IoT System

LEVEL: IOT

DESCRIPTION Local automation is retained while IoT is used for monitoring. This teaches that an IoT device should not always depend on the cloud for basic control.

## Learning Objectives

Combine autonomous local control with connectivity.

Report actuator state.

Explain resilience when the network fails.

## Student Challenge

Control the fan locally from DHT11 readings while publishing temperature, humidity and fan state to the dashboard. Disconnect the network and observe whether local control continues.

## Expected Evidence / Output

Working local controller

IoT dashboard

Network-disconnection test

## Extension Challenge

Allow a temporary remote override with automatic timeout.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 26: IoT Occupancy Monitor

LEVEL: IOT

DESCRIPTION Local object/occupancy detection becomes a remotely visible status service.

## Learning Objectives

Publish event/state data.

Avoid unnecessary repeated messages.

Design a simple presence-monitoring application.

## Student Challenge

Send Available/Occupied state only when the state changes, and display the latest state remotely.

## Expected Evidence / Output

Occupancy dashboard

State-change publishing logic

Message-count comparison with continuous publishing

## Extension Challenge

Record occupancy duration.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 27: IoT Alert System

LEVEL: IOT

DESCRIPTION Students send notifications only when meaningful events occur rather than streaming everything continuously.

## Learning Objectives

Implement event-driven IoT.

Define alert conditions.

Reduce unnecessary communication.

## Student Challenge

Generate a remote alert when environmental values enter a Critical state. Avoid repeatedly sending the same alert while the condition remains unchanged.

## Expected Evidence / Output

Alert demonstration

Event logic

Record of normal-to-critical and critical-to-normal transitions

## Extension Challenge

Add acknowledgement and escalation behavior.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 28: Environmental Data Logger

LEVEL: IOT

DESCRIPTION Students create a historical IoT dataset that will later support analysis and machine learning.

## Learning Objectives

Collect structured time-series data.

Choose a sampling interval.

Visualize trends and identify patterns.

## Student Challenge

Log temperature and humidity over a meaningful period, then plot or inspect the trend and identify at least three observations.

## Expected Evidence / Output

CSV/database/dashboard history

Sampling plan

Three evidence-based observations

## Extension Challenge

Compare different sampling intervals and storage requirements.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 29: Smart Parking Space

LEVEL: IOT

DESCRIPTION Students apply IoT principles to a recognizable smart-city problem.

## Learning Objectives

Translate a real problem into sensing states.

Publish availability information.

Design a user-facing status.

## Student Challenge

Build one parking-space prototype using IR detection. Publish Free/Occupied status and show a clear visual indicator locally and remotely.

## Expected Evidence / Output

Parking prototype

Remote availability display

Test record of arrivals/departures

## Extension Challenge

Expand to multiple spaces and calculate total free spaces.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 30: Multi-Node Monitoring

LEVEL: IOT

DESCRIPTION Students move from one device to a small distributed IoT system and learn the need to identify data sources.

## Learning Objectives

Understand multiple IoT nodes.

Use device identifiers/topics.

Compare measurements from different locations.

## Student Challenge

Connect two or more sensor nodes and show their readings separately on one dashboard. Identify each node clearly and compare their measurements.

## Expected Evidence / Output

Multi-node dashboard

Device identification scheme

Comparison of node readings

## Extension Challenge

Detect and display an offline node.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Edge AI Level

Focus: datasets, machine learning concepts, TinyML and local inference.

# Challenge 31: Build a Sensor Dataset

LEVEL: EDGE AI

DESCRIPTION Students begin the Edge AI workflow by converting sensor observations into labelled examples suitable for model development.

## Learning Objectives

Understand samples, features and labels.

Collect balanced examples.

Document data-collection conditions.

## Student Challenge

Collect environmental samples and label them using agreed classes such as Normal, Warm and Critical. Aim for representative data rather than many identical samples.

## Expected Evidence / Output

Labelled dataset

Class counts

Description of collection procedure

## Extension Challenge

Collect data in different locations/times to improve diversity.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 32: Data Cleaning Challenge

LEVEL: EDGE AI

DESCRIPTION Students inspect their dataset before training and identify values that could mislead a model.

## Learning Objectives

Recognize missing, duplicated and unrealistic data.

Apply simple cleaning rules.

Keep a record of changes.

## Student Challenge

Find and handle missing values, repeated records, impossible readings and obvious sensor errors. Compare dataset size before and after cleaning.

## Expected Evidence / Output

Cleaned dataset

Cleaning log

Before/after summary

## Extension Challenge

Compare deleting bad samples with replacing/imputing values.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 33: Feature Selection

LEVEL: EDGE AI

DESCRIPTION Students decide which measured variables are useful for the target prediction rather than automatically using every available value.

## Learning Objectives

Understand the meaning of a feature.

Compare informative and weak features.

Relate sensor choice to model purpose.

## Student Challenge

Evaluate whether temperature, humidity, occupancy or combinations of them are useful for the selected classification problem. Justify the final feature set.

## Expected Evidence / Output

Feature table

Selected feature set

Written justification

## Extension Challenge

Create simple derived features such as change over time.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 34: Rule-Based Logic vs AI

LEVEL: EDGE AI

DESCRIPTION Students solve the same problem using explicit thresholds and a trained model, then compare the two approaches.

## Learning Objectives

Distinguish programmed rules from learned patterns.

Evaluate both methods on the same test data.

Discuss maintainability and adaptability.

## Student Challenge

Build a threshold classifier and a simple ML classifier for the same classes. Test both using unseen samples and compare results.

## Expected Evidence / Output

Two implementations

Comparison table

Short conclusion on when each approach is preferable

## Extension Challenge

Include memory, speed or explainability in the comparison.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 35: Environmental Condition Classifier

LEVEL: EDGE AI

DESCRIPTION Students train a small model to predict an environmental category from sensor features.

## Learning Objectives

Train a basic classifier.

Separate training and testing data.

Interpret predicted classes.

## Student Challenge

Train a model for classes such as Comfortable/Uncomfortable or Normal/Warning/Critical. Evaluate it before deployment.

## Expected Evidence / Output

Trained model

Test results

Confusion matrix or simple correct/incorrect summary

## Extension Challenge

Experiment with a different model or feature combination.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 36: Edge Inference

LEVEL: EDGE AI

DESCRIPTION Students deploy a trained model to a supported edge device so new sensor measurements are classified locally.

## Learning Objectives

Understand inference versus training.

Deploy a compact model.

Run predictions without cloud dependence.

## Student Challenge

Load the trained model onto the edge device, read live sensor values and print the predicted class locally. Test with multiple real conditions.

## Expected Evidence / Output

On-device inference demonstration

Prediction log

Description of memory/processing constraints encountered

## Extension Challenge

Measure approximate inference time.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 37: Smart Fan with Edge AI

LEVEL: EDGE AI

DESCRIPTION The prediction from the local ML model directly controls a physical actuator, creating an intelligent cyber-physical system.

## Learning Objectives

Connect inference to actuation.

Define safe actions for predicted classes.

Handle uncertain or invalid predictions.

## Student Challenge

Use the model output to select fan OFF/LOW/HIGH behavior. Include a safe fallback if a prediction cannot be produced.

## Expected Evidence / Output

Working AI-controlled fan

Class-to-action table

Ten live prediction/action tests

## Extension Challenge

Compare the AI controller with the earlier threshold controller.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 38: Edge Anomaly Detection

LEVEL: EDGE AI

DESCRIPTION Instead of predicting predefined classes, students identify observations that differ from normal behavior.

## Learning Objectives

Understand anomaly detection.

Establish normal behavior.

Trigger local alerts for unusual data.

## Student Challenge

Collect normal environmental data, define or train an anomaly detector, and test it using deliberately unusual but safe input conditions.

## Expected Evidence / Output

Normal baseline dataset

Anomaly detection results

False alarm discussion

## Extension Challenge

Send only anomalies to the cloud.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 39: Intelligent Occupancy Detection

LEVEL: EDGE AI

DESCRIPTION Students improve simple IR occupancy logic by considering patterns across time or multiple signals rather than one instantaneous reading.

## Learning Objectives

Create time-window features.

Recognize patterns in event data.

Evaluate occupancy predictions.

## Student Challenge

Collect sequences of IR events and optionally environmental signals, label occupancy states and build a simple classifier or pattern-based detector.

## Expected Evidence / Output

Labelled sequence data

Occupancy model/algorithm

Test accuracy summary

## Extension Challenge

Compare single-sensor and multi-sensor performance.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 40: Model Accuracy Test

LEVEL: EDGE AI

DESCRIPTION Students systematically evaluate an Edge AI model using unseen examples and identify the types of mistakes it makes.

## Learning Objectives

Calculate basic accuracy.

Identify false positives and false negatives.

Use evaluation results to improve a system.

## Student Challenge

Prepare an unseen test set, run predictions, count correct and incorrect outputs, calculate accuracy and discuss the most important errors.

## Expected Evidence / Output

Test dataset

Prediction/results table

Accuracy calculation and error discussion

## Extension Challenge

Add precision/recall or per-class accuracy for stronger students.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Advanced Level

Focus: complete intelligent Edge-IoT architectures, optimization, privacy, resilience and prediction.

# Challenge 41: Edge-First Environmental Monitor

LEVEL: ADVANCED

DESCRIPTION Students redesign an IoT system so raw sensor data is processed locally and only useful information is transmitted.

## Learning Objectives

Explain edge-first architecture.

Reduce raw-data transmission.

Separate local intelligence from remote monitoring.

## Student Challenge

Classify environmental conditions locally and send only the class, important summary values or exceptional events to the IoT service.

## Expected Evidence / Output

Architecture diagram

Working edge-first prototype

Comparison of messages sent before and after edge processing

## Extension Challenge

Periodically send a compact summary for long-term trends.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 42: Smart Alert Filtering

LEVEL: ADVANCED

DESCRIPTION Students reduce alert fatigue by using local intelligence to decide whether an event is meaningful before notifying a remote user.

## Learning Objectives

Filter noisy events.

Design persistence/cooldown logic.

Evaluate false alerts.

## Student Challenge

Create local logic or a model that requires an abnormal condition to persist or reach sufficient confidence before an IoT alert is sent.

## Expected Evidence / Output

Filtered alert system

Alert rules/model description

Before/after false-alert comparison

## Extension Challenge

Add multiple alert severity levels.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 43: Cloud vs Edge Experiment

LEVEL: ADVANCED

DESCRIPTION Students implement equivalent decision logic in two architectures and evaluate practical differences.

## Learning Objectives

Compare edge and cloud computing.

Measure latency and network dependency.

Discuss privacy, bandwidth and resilience.

## Student Challenge

Run one version where the decision is local and another where data must be sent remotely before action. Compare response time and behavior when connectivity is removed.

## Expected Evidence / Output

Two architecture diagrams

Latency/connectivity test table

Written comparison

## Extension Challenge

Estimate data usage for both architectures.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 44: Intelligent Smart Room

LEVEL: ADVANCED

DESCRIPTION Students combine environmental and occupancy information to control several room states intelligently.

## Learning Objectives

Perform multi-feature decision-making.

Coordinate multiple outputs.

Design an integrated smart environment.

## Student Challenge

Use temperature, humidity and occupancy to determine room condition and control representative fan/vent/light outputs while reporting status remotely.

## Expected Evidence / Output

Integrated prototype

Decision/model description

Scenario-based test results

## Extension Challenge

Add user preference or time-of-day as another feature.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 45: Predictive Ventilation

LEVEL: ADVANCED

DESCRIPTION Students move from reacting to current conditions toward predicting a near-future need using recent measurements.

## Learning Objectives

Create temporal features.

Understand simple prediction.

Act before a threshold is crossed.

## Student Challenge

Use a short history of temperature/humidity readings to estimate whether ventilation will soon be required. Compare predictive action with purely reactive control.

## Expected Evidence / Output

Historical feature dataset

Prediction method/model

Comparison of predictive and reactive behavior

## Extension Challenge

Tune the prediction horizon and measure usefulness.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 46: Edge Fault Detector

LEVEL: ADVANCED

DESCRIPTION Students distinguish genuine environmental changes from sensor/device faults.

## Learning Objectives

Recognize fault signatures.

Design plausibility checks or anomaly models.

Create safe fallback behavior.

## Student Challenge

Detect conditions such as constant stuck values, impossible readings or unrealistically fast changes. Mark the sensor as Fault rather than treating the reading as real.

## Expected Evidence / Output

Fault-detection logic/model

Injected-fault test cases

Safe fallback action

## Extension Challenge

Publish a maintenance alert with fault type.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 47: Adaptive Motor Controller

LEVEL: ADVANCED

DESCRIPTION Students allow motor behavior to change according to inferred operating conditions rather than fixed user-defined speed values.

## Learning Objectives

Use model output for continuous/discrete control.

Balance responsiveness and stability.

Evaluate actuator behavior.

## Student Challenge

Design an intelligent controller that selects motor speed from environmental/operational features. Test stability when inputs fluctuate.

## Expected Evidence / Output

Adaptive motor prototype

Control strategy

Stability test observations

## Extension Challenge

Add smoothing or confidence-based speed changes.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 48: Bandwidth Saving Challenge

LEVEL: ADVANCED

DESCRIPTION Students quantify the communication benefit of edge processing.

## Learning Objectives

Measure message/data volume.

Design event/summary-based transmission.

Relate architecture to bandwidth cost.

## Student Challenge

Compare three strategies: send every sample, send periodic summaries, and send only state changes/anomalies. Count messages or bytes over the same test period.

## Expected Evidence / Output

Three transmission implementations or simulations

Bandwidth comparison table

Recommendation with justification

## Extension Challenge

Include estimated battery/energy implications.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 49: Edge Privacy Challenge

LEVEL: ADVANCED

DESCRIPTION Students examine which data genuinely needs to leave a device and redesign the architecture to minimize unnecessary exposure.

## Learning Objectives

Apply data-minimization principles.

Separate raw data from derived information.

Explain privacy benefits of local processing.

## Student Challenge

For a selected IoT scenario, list all collected data, decide what remains local and what may be transmitted, then implement or diagram the privacy-aware data flow.

## Expected Evidence / Output

Data classification table

Privacy-aware architecture diagram

Justification for each transmitted field

## Extension Challenge

Add retention/deletion rules for stored data.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Challenge 50: Energy-Aware Intelligent IoT Node

LEVEL: ADVANCED

DESCRIPTION Students optimize sensing, processing and communication so the node does not perform expensive operations unnecessarily.

## Learning Objectives

Identify major energy-consuming operations.

Use event-driven or scheduled behavior.

Balance responsiveness and efficiency.

## Student Challenge

Design a node that samples, infers and communicates only as often as necessary. Compare its activity count with an always-active design.

## Expected Evidence / Output

Energy-aware logic

Activity/communication comparison

Explanation of trade-offs

## Extension Challenge

Use sleep modes if supported by the chosen hardware.

## Student Reflection

1. What worked correctly in your system?

2. What problem did you encounter and how did you solve it?

3. What would you improve in the next version?

# Suggested Assessment Rubric

| Criterion | Weight | Evidence expected |
| --- | --- | --- |
| Functionality | 30% | The prototype performs the required sensing, decision and actuation reliably. |
| Circuit / implementation | 15% | Correct and safe connections; organized hardware implementation. |
| Programming / model | 20% | Clear logic, readable code and correct use of the model where applicable. |
| Testing | 15% | Systematic test cases with recorded results. |
| Design reasoning | 10% | Student can justify thresholds, features, architecture and trade-offs. |
| Documentation / demonstration | 10% | Clear circuit/architecture diagram, code/data and effective demonstration. |